\DOC UNFOLD_CONV

\TYPE {UNFOLD_CONV : (thm list -> conv)}

\SYNOPSIS

Expands sub-components of a hardware description using their definitions.

\LIBRARY unwind

\DESCRIBE

{UNFOLD_CONV thl "t1 /\ ... /\ tn"} returns a theorem of the form:

{

B |- t1 /\ ... /\ tn = t1' /\ ... /\ tn'

}

where each {ti'} is the result of rewriting {ti} with the theorems in

{thl}. The set of assumptions {B} is the union of the instantiated assumptions

of the theorems used for rewriting. If none of the rewrites are applicable to

a {ti}, it is unchanged.

\FAILURE

Never fails.

\EXAMPLE

{

#UNFOLD_CONV [ASSUME "!in out. INV (in,out) = !(t:num). out t = ~(in t)"]

# "INV (l1,l2) /\ INV (l2,l3) /\ (!(t:num). l1 t = l2 (t-1) \/ l3 (t-1))";;

. |- INV(l1,l2) /\ INV(l2,l3) /\ (!t. l1 t = l2(t - 1) \/ l3(t - 1)) =

(!t. l2 t = ~l1 t) /\

(!t. l3 t = ~l2 t) /\

(!t. l1 t = l2(t - 1) \/ l3(t - 1))

}

\SEEALSO

UNFOLD_RIGHT_RULE.

\ENDDOC